# Craig Ssemakula

# Internetworking

# 11th November,2025

# Spanning Tree Protocol (STP) Lab Synopsis

# Key Learnings

## 1. STP Port States & Root Bridge Selection

- STP elects a root bridge based on the lowest bridge ID

- Port states: Forwarding (FWD) allows traffic; Blocking (BLK) prevents loops

- One port is blocked in redundant topologies to create a loop-free logical path

## 2. Loop Prevention Mechanism

- STP blocks redundant paths to prevent broadcast storms and MAC table instability

- Orange link lights in Packet Tracer indicate blocked ports

## 3. STP Convergence Process

- When a link fails, STP recalculates and transitions blocked ports through states: BLK → LIS → LRN → FWD

- After 30-50 seconds, theconnectivity is restored through alternate paths.

# Conclusion

This lab demonstrated how STP is essential for network stability in redundant topologies. By  blocking ports and  recalculating paths during failures, STP ensures both loop prevention and high availability.