COMP 4358 — WIRELESS NETWORKING

TechNova Enterprise

Wireless Network

Build Guide

Dual-core HA · OSPF · WPA2-Enterprise · 7 APs

| Project | TechNova Inc. 3-story office wireless deployment |
| --- | --- |
| Student | Craig Ssendiwala |
| Course | COMP 4358 — Wireless Networking |
| Professor | Azamat Zhamanov |
| Deliverables | Packet Tracer (.pkt) file + PDF report |
| This document | Construction manual — how to actually build it |

Companion files: TechNova_IPPlan.xlsx · technova_enterprise.html

Contents

After opening this file in Word, right-click the table of contents and choose Update Field → Update entire table to populate page numbers.

# 1 · Overview

## 1.1 What this document is

This is a construction manual for the TechNova wireless network. It tells you which cables to plug in, which commands to paste, and which outputs to expect — in the order you should actually do them. The goal is a Packet Tracer file that a network engineer would look at and nod.

Read Chapters 1 and 2 end to end before you open Packet Tracer. After that, work one phase at a time (Chapter 3). Each phase ends with a verification step so you catch mistakes when they are small and cheap, not at the end when they are buried under twelve other configs.

## 1.2 What you are building

A realistic enterprise wireless deployment for a fictional 3-story office called TechNova Inc. The design has four pillars:

- A dual-core Layer-3 distribution with HSRP-protected gateways and LACP EtherChannels, so no single switch failure takes down a VLAN.

- OSPF between the edge router and both cores, so the Internet path reconverges automatically around failures.

- Centralized wireless using a Wireless LAN Controller with seven lightweight APs registered via CAPWAP, authenticated with WPA2-Enterprise against a RADIUS server.

- A hardened management plane: SSH only, authenticated NTP, centralized Syslog, port security, BPDU Guard, and ACLs that isolate the Guest VLAN from everything internal.

## 1.3 Requirements coverage

Every mandatory requirement from the COMP 4358 brief is satisfied, and every optional advanced feature listed in Section 3.5 of the brief is implemented. Nothing in this build defies the scenario.

| Requirement (from brief) | How this build satisfies it |
| --- | --- |
| 3+ APs, one per floor | 7 APs total: 2 on F1, 3 on F2, 2 on F3 |
| 1+ WLC with central management | WLC-Main with all 7 APs joined via CAPWAP |
| 3 SSIDs with correct security | TechNova-Employee (WPA2-Enterprise / 802.1X), TechNova-Guest (Open), TechNova-IoT (WPA2-PSK, SSID hidden) |
| 4 VLANs with specified IDs | V10 Employee, V20 Guest, V30 IoT, V99 Management |
| Channels 1/6/11 + UNII on 5 GHz | 1,6,11 across 2.4 GHz APs; UNII-1 and UNII-3 on the 5 GHz APs |
| WPA2 with AES | Enabled on Employee (WPA2-Enterprise) and IoT (WPA2-PSK) SSIDs |
| VLAN segmentation | Layer-2 separation plus Layer-3 ACLs enforced at the VLAN SVIs |
| ACL isolating Guest from internal | Extended ACL applied inbound on the VLAN 20 SVI |
| Hidden SSID on IoT network | Broadcast SSID disabled on WLAN 3 (TechNova-IoT) |
| DHCP + DNS (mandatory base) | SRV-Core runs four DHCP pools and a DNS zone |
| 2+ additional features | Inter-VLAN routing and Port Security are the required two; added NTP, Syslog, AAA/RADIUS, SSH, and BPDU Guard as extra credit |
| Bluetooth conceptual component | Covered in the PDF report (Section 4.7), not in the .pkt file |

## 1.4 How each phase is structured

Every phase in Chapter 3 uses the same five-part layout, so you always know what to look for:

- Goal — one sentence on what the phase achieves.

- Prerequisites — what must already be working before you start.

- Steps — GUI actions and CLI commands in the exact order to run them.

- Verify — one or two commands whose output proves the phase is complete.

- Common trip-ups — the one or two things students get wrong in this phase.

Tip Keep the companion spreadsheet (TechNova_IPPlan.xlsx) open on a second monitor while building. The build guide tells you what to type; the spreadsheet tells you which value goes where.

# 2 · Pre-flight

## 2.1 Software and file setup

- Install Cisco Packet Tracer 8.2 or newer. Older versions lack reliable HSRP and 802.1X support on the 3560 and 2911 platforms this build uses.

- Create a working folder, for example C:\NAU\COMP4358\FinalProject.

- Open Packet Tracer. File → Save As. Save immediately as Ssendiwala_Craig_COMP4358_FinalProject.pkt.

- Set File → Preferences → Interface → Autosave to 5 minutes. Packet Tracer crashes unforgivingly, and autosave is your safety net.

- Copy the companion spreadsheet and topology mockup into the same folder so everything is in one place.

Watch out Get the filename right from the start. The brief imposes a 5% penalty for wrong filenames. The exact format is LastName_FirstName_COMP4358_FinalProject.pkt — rename the file before you start rather than trying to rename it later.

## 2.2 Build workflow at a glance

The build is divided into ten phases with verification built into each. Do not jump ahead. A failing verification is the only legitimate reason to stop — fix it before moving on.

| Phase | What gets built | Approx time |
| --- | --- | --- |
| 1 | Device placement and cabling (no config yet) | 45 min |
| 2 | Base config — hostnames, mgmt IPs, SSH, banner | 45 min |
| 3 | Layer 2 — VLANs, trunks, EtherChannel | 60 min |
| 4 | Layer 3 core — SVIs, HSRP, STP priorities | 60 min |
| 5 | Edge router — OSPF, NAT, default route | 30 min |
| 6 | Services — DHCP, DNS, RADIUS, NTP, Syslog | 45 min |
| 7 | Wireless — WLC setup, AP join, SSIDs, 802.1X | 75 min |
| 8 | Security hardening — ACLs, port security, BPDU guard | 30 min |
| 9 | Client association and end-to-end testing | 30 min |
| 10 | Verification commands and screenshot capture | 45 min |

Total budget: about 8 hours of focused work. Spread it across three sessions. Trying to do this in one sitting is how mistakes pile up and compound.

## 2.3 Companion files in your folder

- TechNova_IPPlan.xlsx — seven sheets covering VLANs, devices, IP addresses, port map, channel plan, and verification tests.

- technova_enterprise.html — visual mockup of the completed topology (open in any browser).

- This document — the construction manual you are reading now.

## 2.4 Conventions in this guide

CLI commands appear in a gray monospace block, ready to paste into Packet Tracer. GUI actions appear as numbered steps. Yellow callout boxes flag things that routinely break. Green boxes are optional polish that strengthens the PDF report. When you see a banner like the one below, paste commands line by line — never paste the whole block in one go, as Packet Tracer occasionally drops characters on long paste buffers.

SW-Core-1(config)# interface vlan 10

SW-Core-1(config-if)#  ip address 192.168.10.2 255.255.255.0

# 3 · Build phases

Ten phases, each with a clear start state and a clear end state. Work them in order. When a phase's verification command shows the expected output, move on — do not tinker.

## Phase 1 — Device placement and cabling

### Goal

Drop every device into the Logical Workspace, label them exactly as specified, and connect every cable. No configuration in this phase — get the physical topology correct first.

### Prerequisites

Fresh Packet Tracer file, saved with the correct name.

### Steps

#### Device drop list

From the bottom-right panel in Packet Tracer, drag the following devices onto the workspace. Place them roughly in the layout shown in the topology mockup (internet at top, cores in the middle, floors stacked below).

| Category | PT model | Qty | Names to assign |
| --- | --- | --- | --- |
| Router | 2911 | 1 | R-Edge |
| L3 Switch | 3560-24PS | 2 | SW-Core-1, SW-Core-2 |
| L2 Switch | 2960-24TT | 3 | SW-F1, SW-F2, SW-F3 |
| Wireless LAN Controller | WLC3504 | 1 | WLC-Main |
| Lightweight AP | LAP-PT | 7 | AP-F1-A/B, AP-F2-A/B/C, AP-F3-A/B |
| Server | Server-PT | 2 | SRV-Core, SRV-AAA |
| Cloud | Cloud-PT | 1 | ISP-Cloud |
| Laptop | Laptop-PT | 6 | Laptop-IT-01, Laptop-Eng-01/02, Laptop-HR-01, Laptop-Exec-01, Tablet-Conf-01 (use Tablet-PT) |
| Smartphone | Smartphone-PT | 1 | Phone-Guest-Lobby |
| IoT | Smart Sensor / IP Camera | 2 | IoT-Sensor-Exec, IPCam-Training |

Tip Label devices as you drop them, not after. Click the device, and on the Config tab set Display Name to the exact string above. Names in the topology screenshot are graded for clarity per the rubric.

#### Port plan at a glance

Read this once before cabling and again before writing CLI. The 3560-24PS only has two gigabit ports (gi0/1, gi0/2) — they are reserved for the R-Edge uplinks. Every inter-switch link on the cores uses FastEthernet. The 2960-24TT floor switches use the same convention to keep everything consistent.

| Device | Gigabit ports | FastEthernet usage |
| --- | --- | --- |
| SW-Core-1 | gi0/1 → R-Edge (OSPF L3); gi0/2 unused | fa0/1-2 = Po10 to SW-F1; fa0/3 = F2 backup trunk; fa0/5-6 = Po30 to SW-F3; fa0/10 = SRV-Core; fa0/11 = SRV-AAA; fa0/23-24 = Po1 to SW-Core-2 |
| SW-Core-2 | gi0/1 → R-Edge (OSPF L3); gi0/2 → WLC-Main (trunk) | fa0/1 = F1 backup trunk; fa0/3-4 = Po20 to SW-F2; fa0/5 = F3 backup trunk; fa0/23-24 = Po1 to SW-Core-1 |
| SW-F1 / SW-F2 / SW-F3 | gi0/1-2 unused (future expansion) | fa0/1-5 = AP uplinks (V99 access); fa0/22 = backup trunk to the other core; fa0/23-24 = primary Po to home core |

#### Cable list

All cables are Copper Straight-Through unless noted (switch-to-switch links use Copper Cross-Over). Use the Port Map sheet in the companion spreadsheet as your running checklist — tick each cable as you run it.

Port naming reminder: on the 3560-24PS cores, only gi0/1 and gi0/2 are gigabit; all other 24 ports are FastEthernet (fa0/1–fa0/24). The gig ports are reserved for the R-Edge uplinks; every other inter-switch link runs on FastEthernet.

| From | Port | To | Port | Cable | Purpose |
| --- | --- | --- | --- | --- | --- |
| ISP-Cloud | Fa0 | R-Edge | gi0/0 | straight | Simulated ISP uplink |
| R-Edge | gi0/1 | SW-Core-1 | gi0/1 | straight | Routed L3 link (OSPF) |
| R-Edge | gi0/2 | SW-Core-2 | gi0/1 | straight | Routed L3 link (OSPF) |
| SW-Core-1 | fa0/23 | SW-Core-2 | fa0/23 | cross-over | Po1 inter-core member 1 (LACP) |
| SW-Core-1 | fa0/24 | SW-Core-2 | fa0/24 | cross-over | Po1 inter-core member 2 (LACP) |
| SW-Core-1 | fa0/1 | SW-F1 | fa0/23 | cross-over | Po10 primary uplink F1 (member 1) |
| SW-Core-1 | fa0/2 | SW-F1 | fa0/24 | cross-over | Po10 primary uplink F1 (member 2) |
| SW-Core-2 | fa0/1 | SW-F1 | fa0/22 | cross-over | F1 backup trunk |
| SW-Core-2 | fa0/3 | SW-F2 | fa0/23 | cross-over | Po20 primary uplink F2 (member 1) |
| SW-Core-2 | fa0/4 | SW-F2 | fa0/24 | cross-over | Po20 primary uplink F2 (member 2) |
| SW-Core-1 | fa0/3 | SW-F2 | fa0/22 | cross-over | F2 backup trunk |
| SW-Core-1 | fa0/5 | SW-F3 | fa0/23 | cross-over | Po30 primary uplink F3 (member 1) |
| SW-Core-1 | fa0/6 | SW-F3 | fa0/24 | cross-over | Po30 primary uplink F3 (member 2) |
| SW-Core-2 | fa0/5 | SW-F3 | fa0/22 | cross-over | F3 backup trunk |
| SW-Core-2 | gi0/2 | WLC-Main | GigabitEthernet1 | straight | Trunk to WLC (gig-to-gig) |
| SW-Core-1 | fa0/10 | SRV-Core | fa0 | straight | Server access V99 |
| SW-Core-1 | fa0/11 | SRV-AAA | fa0 | straight | Server access V99 |
| SW-F1 | fa0/1..2 | AP-F1-A/B | GigabitEthernet0 | straight | AP uplinks |
| SW-F2 | fa0/1..3 | AP-F2-A/B/C | GigabitEthernet0 | straight | AP uplinks |
| SW-F3 | fa0/1..2 | AP-F3-A/B | GigabitEthernet0 | straight | AP uplinks |

### Verify

At the end of Phase 1, all link lights should be amber (Layer 2 up). Link lights will go green after Phase 3 when trunking is configured. Save the file.

### Common trip-ups

- Dropping a regular Switch 2960 where you needed a 3560 L3. If your core switch does not show the 'no switchport' option later, you have the wrong model — delete and replace.

- Plugging AP uplinks into access ports on the wrong floor switch. Cross-check the port map.

- Forgetting the second link of an EtherChannel. Every Po has exactly two member links; if you see only one, you will get channel-group flapping later.

## Phase 2 — Base configuration

### Goal

Every network device has its hostname, management IP, local user, SSH, and banner set. Telnet is disabled everywhere. This is the foundation that every later phase relies on.

### Prerequisites

Phase 1 complete. All devices dropped and cabled.

### Steps

Open the CLI tab on R-Edge and paste these commands line by line:

#### R-Edge (router)

enable

configure terminal

hostname R-Edge

ip domain-name technova.local

username admin privilege 15 secret Techn0va!

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 4

login local

transport input ssh

exec-timeout 10 0

line console 0

logging synchronous

exec-timeout 15 0

service password-encryption

no ip http server

banner motd # Authorized access only. TechNova Inc. #

end

write memory

#### SW-Core-1 (and identical on SW-Core-2, SW-F1, SW-F2, SW-F3 with their own hostname)

Repeat these commands on every switch, substituting the hostname each time. Values of domain-name, username, and banner stay identical across all five switches.

enable

configure terminal

hostname SW-Core-1

ip domain-name technova.local

username admin privilege 15 secret Techn0va!

crypto key generate rsa modulus 1024

ip ssh version 2

line vty 0 15

login local

transport input ssh

line console 0

logging synchronous

service password-encryption

banner motd # Authorized access only. TechNova Inc. #

end

write memory

Context The mgmt IP address itself is applied in Phase 3 (on VLAN 99 for switches) or Phase 5 (for R-Edge). Phase 2 only sets up authentication and remote access.

### Verify

show running-config | include hostname|username|ssh|transport

You should see the hostname, one local user, SSH version 2, and 'transport input ssh' on the VTY lines — no Telnet.

### Common trip-ups

- Skipping 'ip domain-name' before generating the RSA key — the key generation will fail silently.

- Copying the whole block into the console on a real 2960. Packet Tracer handles big pastes on routers but sometimes skips lines on switches. Paste in chunks of 5–6 lines.

## Phase 3 — Layer 2 (VLANs, trunks, EtherChannel)

### Goal

Every switch has the four VLANs defined. Every inter-switch link is either an 802.1Q trunk or a LACP EtherChannel carrying the right VLANs.

### Prerequisites

Phase 2 complete on all five switches.

### Steps

#### Define VLANs on all 5 switches

Run this block on SW-Core-1, SW-Core-2, SW-F1, SW-F2, SW-F3. Identical on all five.

configure terminal

vlan 10

name Employee

vlan 20

name Guest

vlan 30

name IoT

vlan 99

name Management

end

write memory

#### Inter-core EtherChannel (Po1) — run on SW-Core-1

configure terminal

interface range fa0/23 - 24

description TO-SW-CORE-2-Po1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-protocol lacp

channel-group 1 mode active

interface port-channel 1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

end

Repeat the same block on SW-Core-2 (the description changes to TO-SW-CORE-1, everything else is identical).

Watch out Both ends of a LACP EtherChannel must be 'mode active'. If one side is 'passive' or 'desirable', the bundle will not form. Never mix modes.

#### Floor uplink EtherChannels

Floor 1 primary uplink to Core-1 as Po10. Floor 2 primary uplink to Core-2 as Po20. Floor 3 primary uplink to Core-1 as Po30. Each Po is a trunk carrying all four VLANs.

On SW-F1:

configure terminal

interface range fa0/23 - 24

description TO-CORE-1-Po10

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 10 mode active

interface port-channel 10

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

!

interface fa0/22

description TO-CORE-2-BACKUP

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

end

On SW-Core-1, the matching end:

configure terminal

interface range fa0/1 - 2

description TO-SW-F1-Po10

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 10 mode active

interface port-channel 10

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

end

Floor 2 (Po20 on Core-2, core ports fa0/3-4) and Floor 3 (Po30 on Core-1, core ports fa0/5-6) use the identical pattern — just change the channel-group number and the physical port range to match the Port Map.

#### Access ports for APs and servers

All AP uplinks and server connections go into VLAN 99 as access ports. On each floor switch:

configure terminal

interface range fa0/1 - 5

description AP-ACCESS-V99

switchport mode access

switchport access vlan 99

spanning-tree portfast

spanning-tree bpduguard enable

end

On SW-Core-1, add the two server ports:

configure terminal

interface fa0/10

description TO-SRV-CORE

switchport mode access

switchport access vlan 99

spanning-tree portfast

spanning-tree bpduguard enable

interface fa0/11

description TO-SRV-AAA

switchport mode access

switchport access vlan 99

spanning-tree portfast

spanning-tree bpduguard enable

end

write memory

### Verify

show etherchannel summary

Every Po should show state 'SU' (bundled in L2) with two 'P' (bundled) ports. Also:

show interfaces trunk

All trunks should show allowed VLANs 10, 20, 30, 99 and active encapsulation dot1q.

### Common trip-ups

- 'show etherchannel summary' shows the Po but 'SD' (down) — means no link on the member ports. Re-cable.

- VLANs 10/20/30/99 not shown in the trunk allowed list — check 'switchport trunk allowed vlan 10,20,30,99' was applied to both the Po interface AND the member physical interfaces. Inconsistency causes native-VLAN mismatch warnings.

## Phase 4 — Layer 3 core (SVIs, HSRP, STP)

### Goal

Both core switches have SVIs for all four VLANs. HSRP is split-active: Core-1 is active for VLANs 10 and 30, Core-2 is active for VLANs 20 and 99. STP root bridge placement matches the HSRP active side so traffic does not cross the inter-core link unnecessarily.

### Prerequisites

Phase 3 complete. Trunks and EtherChannels up.

### Steps

#### Enable L3 routing and set STP priorities on SW-Core-1

configure terminal

ip routing

spanning-tree vlan 10,30 priority 4096

spanning-tree vlan 20,99 priority 8192

end

#### On SW-Core-2

configure terminal

ip routing

spanning-tree vlan 20,99 priority 4096

spanning-tree vlan 10,30 priority 8192

end

Context Priority 4096 = root bridge for that VLAN. Priority 8192 = secondary root. Matching HSRP active to STP root keeps the forwarding path optimal — no traffic bounces across the inter-core trunk unnecessarily.

#### SVIs with HSRP on SW-Core-1 (active for V10 + V30, standby for V20 + V99)

configure terminal

interface vlan 10

description Employee-GW-HSRP-Active

ip address 192.168.10.2 255.255.255.0

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

no shutdown

!

interface vlan 20

description Guest-GW-HSRP-Standby

ip address 192.168.20.2 255.255.255.0

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 100

no shutdown

!

interface vlan 30

description IoT-GW-HSRP-Active

ip address 192.168.30.2 255.255.255.0

standby version 2

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

no shutdown

!

interface vlan 99

description Mgmt-GW-HSRP-Standby

ip address 192.168.99.2 255.255.255.0

standby version 2

standby 99 ip 192.168.99.1

standby 99 priority 100

no shutdown

end

write memory

#### SVIs with HSRP on SW-Core-2 (mirror: standby for V10 + V30, active for V20 + V99)

configure terminal

interface vlan 10

ip address 192.168.10.3 255.255.255.0

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 100

no shutdown

!

interface vlan 20

ip address 192.168.20.3 255.255.255.0

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

no shutdown

!

interface vlan 30

ip address 192.168.30.3 255.255.255.0

standby version 2

standby 30 ip 192.168.30.1

standby 30 priority 100

no shutdown

!

interface vlan 99

ip address 192.168.99.3 255.255.255.0

standby version 2

standby 99 ip 192.168.99.1

standby 99 priority 110

standby 99 preempt

no shutdown

end

write memory

#### Routed links to R-Edge (L3 transit)

These are the two gigabit ports on each core, connecting up to R-Edge. On the 3560-24PS, gi0/1 is one of only two gigabit ports — use it here, and keep gi0/2 unused for future expansion.

On SW-Core-1:

configure terminal

interface gi0/1

no switchport

description TO-R-EDGE-OSPF

ip address 10.0.0.2 255.255.255.252

no shutdown

end

On SW-Core-2:

configure terminal

interface gi0/1

no switchport

description TO-R-EDGE-OSPF

ip address 10.0.0.6 255.255.255.252

no shutdown

end

### Verify

show standby brief

Core-1: Active for V10, V30; Standby for V20, V99. Core-2: the opposite. Both columns show the virtual IP ending in .1.

show spanning-tree root

Core-1: root for V10, V30. Core-2: root for V20, V99. Root costs should be 0 on the root bridge for the relevant VLAN.

### Common trip-ups

- HSRP 'Init' state stuck for minutes — check the SVI is 'no shutdown' and the inter-core trunk carries that VLAN.

- 'standby 10 ip' rejected — you forgot 'standby version 2'. Version 2 is required for HSRPv2 which supports subsecond hellos.

- Forgetting 'preempt' on the active priority 110 side. Without preempt, HSRP will not take back active role after a failover — Core-1 would stay standby after recovering.

## Phase 5 — Edge router (OSPF + NAT)

### Goal

R-Edge peers with both cores via OSPF in area 0, learns all internal subnets, and NATs them out to the simulated Internet.

### Prerequisites

Phase 4 complete. Cores have routed links up on 10.0.0.2/30 and 10.0.0.6/30.

### Steps

#### R-Edge interface configuration

configure terminal

interface gi0/0

description WAN-TO-ISP

ip address dhcp

ip nat outside

no shutdown

interface gi0/1

description TO-SW-CORE-1

ip address 10.0.0.1 255.255.255.252

ip nat inside

no shutdown

interface gi0/2

description TO-SW-CORE-2

ip address 10.0.0.5 255.255.255.252

ip nat inside

no shutdown

end

#### OSPF

configure terminal

router ospf 1

router-id 1.1.1.1

passive-interface default

no passive-interface gi0/1

no passive-interface gi0/2

network 10.0.0.0 0.0.0.3 area 0

network 10.0.0.4 0.0.0.3 area 0

default-information originate always

end

#### OSPF on the cores (already cabled, now enable the process)

SW-Core-1:

configure terminal

router ospf 1

router-id 2.2.2.2

passive-interface default

no passive-interface gi0/1

network 10.0.0.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.99.0 0.0.0.255 area 0

end

SW-Core-2 — same, but router-id 3.3.3.3 and network 10.0.0.4 0.0.0.3 area 0.

#### NAT overload (PAT) on R-Edge

configure terminal

access-list 10 permit 192.168.0.0 0.0.255.255

ip nat inside source list 10 interface gi0/0 overload

end

write memory

### Verify

show ip ospf neighbor

On R-Edge: two neighbors, both state FULL, one via gi0/1 (Core-1), one via gi0/2 (Core-2).

show ip route ospf

R-Edge sees all four internal /24 subnets as OSPF routes via both next hops, with equal cost (ECMP).

### Common trip-ups

- Neighbor stuck in EXSTART — MTU mismatch. Check 'ip mtu' on both sides; default is fine in Packet Tracer.

- Default route not propagating — you forgot 'default-information originate always' on R-Edge.

- Cores show OSPF neighbor but clients cannot reach the Internet. Check NAT access-list 10 is applied on R-Edge.

Tip At this point your cores and R-Edge form a fully-converged routed domain. Lose a core and OSPF reconverges in ~10 seconds; lose the HSRP active and clients keep their gateway in under a second. This is the headline story for your report's Section 4.9 (challenges/lessons).

## Phase 6 — Services (DHCP, DNS, RADIUS, NTP, Syslog)

### Goal

SRV-Core offers DHCP for all four VLANs and DNS for the technova.local zone. SRV-AAA runs RADIUS for 802.1X, NTP as a master clock source, and Syslog to collect logs from every network device.

### Prerequisites

Phase 5 complete. Both core switches are reachable from the server segment (VLAN 99).

### Steps

#### SRV-Core static IP and services

- Click SRV-Core → Desktop → IP Configuration. Set Static: IP 192.168.99.40, Mask 255.255.255.0, Gateway 192.168.99.1, DNS 127.0.0.1.

- Services tab → DHCP → On. Create four pools per the table below.

- Services tab → DNS → On. Add an A record: technova.local → 192.168.99.40.

| Pool name | Default gateway | DNS | Start IP | Subnet mask | Max users |
| --- | --- | --- | --- | --- | --- |
| EmployeePool | 192.168.10.1 | 192.168.99.40 | 192.168.10.50 | 255.255.255.0 | 150 |
| GuestPool | 192.168.20.1 | 8.8.8.8 | 192.168.20.50 | 255.255.255.0 | 150 |
| IoTPool | 192.168.30.1 | 192.168.99.40 | 192.168.30.50 | 255.255.255.0 | 50 |
| MgmtPool | 192.168.99.1 | 192.168.99.40 | 192.168.99.50 | 255.255.255.0 | 50 |

Watch out DHCP relay — VLAN 10/20/30 clients live on different subnets from the DHCP server. You MUST add 'ip helper-address 192.168.99.40' to every user-facing SVI on both cores, OR the switches will drop the DHCPDISCOVER broadcasts. The next code block does this.

#### DHCP helper addresses on core switches

Run on both SW-Core-1 and SW-Core-2:

configure terminal

interface vlan 10

ip helper-address 192.168.99.40

interface vlan 20

ip helper-address 192.168.99.40

interface vlan 30

ip helper-address 192.168.99.40

end

write memory

#### SRV-AAA static IP and services

- Click SRV-AAA → Desktop → IP Configuration. Static: IP 192.168.99.41, Mask /24, Gateway 192.168.99.1, DNS 192.168.99.40.

- Services tab → AAA → On. Configure:

  - Client Name: WLC-Main, Client IP: 192.168.99.10, Secret: Radius-TechN0va, ServerType: Radius. Add the WLC as a client.

  - Also add Client: R-Edge, IP 10.0.0.1, same secret (for future admin AAA if desired).

  - User Setup: add at least three users. Suggested: craig.ssendiwala / TNova-C!, alice.it / TNova-A!, bob.eng / TNova-B!.

- Services tab → NTP → On. No other config required in Packet Tracer — it acts as a stratum-1-equivalent source.

- Services tab → Syslog → On.

#### Point the network at the new services

On every switch and router, add NTP and Syslog targets:

configure terminal

ntp server 192.168.99.41

logging host 192.168.99.41

logging trap informational

service timestamps log datetime msec

service timestamps debug datetime msec

end

write memory

### Verify

show ntp associations

You should see 192.168.99.41 reachable with a non-zero reach value.

show logging

Logs should show timestamped messages and the configured trap level.

### Common trip-ups

- DHCP pools created but clients on V10 never get an address. Almost always means the 'ip helper-address' is missing on the VLAN SVI.

- RADIUS server test fails later in Phase 7 — confirm the shared secret matches on both the WLC and the AAA server, and that the WLC's management IP (192.168.99.10) is registered as a Client in the AAA config.

## Phase 7 — Wireless (WLC, APs, SSIDs, 802.1X)

### Goal

The WLC has a management IP on VLAN 99 and interfaces for VLANs 10, 20, 30. All seven APs are associated and visible in the WLC's Wireless tab. Three WLANs are broadcasting with the correct security.

### Prerequisites

Phase 6 complete. DHCP is functional for VLAN 99 so the APs can obtain an IP address.

### Steps

#### WLC-Main management IP and trunk uplink

- Click WLC-Main. Under the Config tab, set the hostname to WLC-Main.

- Click GigabitEthernet1 → Set switchport mode: Trunk, Native VLAN: 99, Allowed VLANs: 10,20,30,99. Use the core's gi0/2 as the other end so both sides run at gigabit speed (mixing fast/gig can break trunking in some PT versions).

- Management interface → IP 192.168.99.10, Mask /24, Gateway 192.168.99.1.

#### Create dynamic interfaces on the WLC for each user VLAN

In the WLC GUI: Controller → Interfaces → New…

| Interface name | VLAN | IP address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| employee | 10 | 192.168.10.5 | 255.255.255.0 | 192.168.10.1 |
| guest | 20 | 192.168.20.5 | 255.255.255.0 | 192.168.20.1 |
| iot | 30 | 192.168.30.5 | 255.255.255.0 | 192.168.30.1 |

#### Add the RADIUS server to the WLC

- Security → AAA → RADIUS → Authentication → New…

- Server IP: 192.168.99.41, Port: 1812, Shared Secret: Radius-TechN0va.

- Check 'Network User' and 'Management'. Save.

#### Create three WLANs (SSIDs)

WLANs → New…

| WLAN ID | SSID (Profile) | Interface | Security (L2) | Broadcast SSID | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | TechNova-Employee | employee | WPA2 / 802.1X (Enterprise) | Enabled | Set AAA Server → 192.168.99.41 on Security → AAA tab |
| 2 | TechNova-Guest | guest | None (Open) | Enabled | Captive portal described in report only |
| 3 | TechNova-IoT | iot | WPA2-PSK | Disabled (hidden) | PSK: IoT-S3nsor-Net! |

Context When configuring WLAN 1 (Employee), on the Security tab change Layer 2 Security to 'WPA+WPA2', then under AAA Servers pick the RADIUS server you added. This is what gives you full 802.1X/WPA2-Enterprise authentication.

#### APs associate with the WLC

In Packet Tracer the LAP-PT joins the WLC automatically once it has IP connectivity on VLAN 99 and can reach the WLC management IP. Click each AP → Config → AP Wireless → ensure the controller IP is set to 192.168.99.10. After a few seconds the AP should show 'Associated'.

#### Channel and band configuration per AP

In the WLC GUI, under Wireless → Access Points → select each AP → Radio → set channel per the Channel Plan sheet. Quick reference:

| AP | Band | Channel | Power |
| --- | --- | --- | --- |
| AP-F1-A | 2.4 GHz | 1 | 17 dBm |
| AP-F1-B | 5 GHz | 36 | 17 dBm |
| AP-F2-A | 2.4 GHz | 6 | 17 dBm |
| AP-F2-B | 5 GHz | 44 | 17 dBm |
| AP-F2-C | 5 GHz | 149 | 20 dBm |
| AP-F3-A | 2.4 GHz | 11 | 17 dBm |
| AP-F3-B | 5 GHz | 157 | 17 dBm |

### Verify

- WLC GUI → Wireless → All APs: all 7 APs show as Associated / Joined.

- WLC GUI → WLANs: three WLANs with correct security icons (padlock for secured, open for guest).

- From a test laptop, join TechNova-Employee with username craig.ssendiwala / TNova-C! → should associate and receive an IP in 192.168.10.x.

### Common trip-ups

- APs never associate — check DHCP is serving V99 and the AP can reach the WLC. 'ping 192.168.99.10' from the AP CLI is a fast sanity check.

- Employee SSID appears but association fails with WPA2-Enterprise. Confirm RADIUS shared secret matches on both sides and the user exists on SRV-AAA.

- Guest SSID not reachable — some PT versions require you to explicitly enable broadcast on the SSID in the WLAN's Advanced tab.

## Phase 8 — Security hardening (ACLs, port security, BPDU Guard)

### Goal

Guest VLAN is isolated from every internal subnet. Access ports enforce MAC-based port security. BPDU Guard protects against accidental STP attacks (already done in Phase 3 but worth verifying).

### Prerequisites

Phase 7 complete. Wireless clients can associate and obtain DHCP.

### Steps

#### ACL to isolate Guest VLAN — apply on BOTH cores

This extended ACL blocks Guest from reaching any internal subnet, but permits Internet-bound traffic.

configure terminal

ip access-list extended GUEST-ISOLATION

remark --- Block Guest from internal VLANs ---

deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

deny ip 192.168.20.0 0.0.0.255 192.168.99.0 0.0.0.255

remark --- Allow Guest to reach the Internet ---

permit ip 192.168.20.0 0.0.0.255 any

!

interface vlan 20

ip access-group GUEST-ISOLATION in

end

write memory

Context Apply the ACL on both SW-Core-1 and SW-Core-2, on each switch's VLAN 20 SVI. Because HSRP presents a single virtual gateway, either core may route Guest traffic at any given moment — both need the ACL.

#### Port security on AP access ports

On each floor switch, harden the AP uplink ports. Run this on SW-F1, SW-F2, SW-F3, adjusting the interface range to match each switch's AP-connected ports.

configure terminal

interface range fa0/1 - 5

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

end

write memory

#### Verify BPDU Guard on access ports

show spanning-tree interface fa0/1 detail

Look for 'The port is in the portfast mode' and 'Bpdu guard is enabled'. If missing, re-run the Phase 3 snippet for PortFast + BPDU Guard.

### Verify

- From Phone-Guest-Lobby, ping 192.168.10.50 → must fail (ACL working).

- From Phone-Guest-Lobby, ping 8.8.8.8 (or the ISP IP) → must succeed.

show port-security interface fa0/1

Port status 'Secure-up', violation mode 'restrict', sticky MAC learned matches the AP's MAC.

### Common trip-ups

- ACL applied on only one core — Guest traffic routed via the other core bypasses the ACL. Apply on both.

- Port Security violation shuts the port — switch 'err-disable' state. Use 'shutdown' then 'no shutdown' on the interface to clear.

## Phase 9 — Client association and end-to-end testing

### Goal

Every wireless client is associated with the correct SSID, has the right IP address, and can reach (or is correctly blocked from reaching) the resources it should. This is where the design is proven.

### Prerequisites

Phase 8 complete. All three SSIDs operating, ACLs in place.

### Steps

#### Join each client to the right SSID

For each client: click the device → Desktop → PC Wireless → Connect. Pick SSID, enter credentials if required.

| Client | SSID | Credentials | Expected subnet |
| --- | --- | --- | --- |
| Laptop-IT-01 | TechNova-Employee | craig.ssendiwala / TNova-C! | 192.168.10.x |
| Laptop-Eng-01 | TechNova-Employee | bob.eng / TNova-B! | 192.168.10.x |
| Laptop-Eng-02 | TechNova-Employee | bob.eng / TNova-B! | 192.168.10.x |
| Tablet-Conf-01 | TechNova-Employee | alice.it / TNova-A! | 192.168.10.x |
| Laptop-HR-01 | TechNova-Employee | alice.it / TNova-A! | 192.168.10.x |
| Laptop-Exec-01 | TechNova-Employee | craig.ssendiwala / TNova-C! | 192.168.10.x |
| Phone-Guest-Lobby | TechNova-Guest | (none, open) | 192.168.20.x |
| IoT-Sensor-Exec | TechNova-IoT | PSK: IoT-S3nsor-Net! | 192.168.30.x |
| IPCam-Training | TechNova-IoT | PSK: IoT-S3nsor-Net! | 192.168.30.x |

Tip On the IoT clients, the SSID is hidden — you will need to type 'TechNova-IoT' manually rather than picking it from a list. This is a good example for your report: hidden SSID is not a real security measure, as the client software still broadcasts the SSID in probe requests.

#### End-to-end reachability tests

Run every test in the Verification sheet of the companion spreadsheet. The key ones:

- Employee laptop → ping 192.168.99.40 (server) → should succeed.

- Employee laptop → ping 8.8.8.8 (simulated Internet) → should succeed.

- Guest phone → ping 8.8.8.8 → should succeed.

- Guest phone → ping 192.168.10.50 → must fail (ACL).

- Guest phone → ping 192.168.99.10 (WLC) → must fail (ACL).

- IoT sensor → ping 192.168.99.40 → should succeed (IoT does need DNS).

### Verify

All 25 tests in the Verification sheet pass. If any fail, back-trace: check DHCP → gateway → inter-VLAN routing → ACL.

### Common trip-ups

- Smartphone-PT sometimes refuses 802.1X in PT. Keep the guest phone on the Guest SSID (which is what the scenario says anyway) and put the Employee 802.1X testing on laptops.

- Ping fails but everything looks configured — run 'tracert' from the client. The last hop reached tells you which device is dropping traffic.

## Phase 10 — Verification commands and screenshots

### Goal

You capture every screenshot required by the brief (Section 4.8) plus a few extras that demonstrate the enterprise-grade features.

### Required screenshots (from the brief)

| # | What to capture | Where to capture it |
| --- | --- | --- |
| 1 | Full network topology | Packet Tracer Logical Workspace, all devices labeled |
| 2 | Wireless SSID configuration | WLC GUI → WLANs list showing 3 SSIDs with security modes |
| 3 | WLC with associated APs | WLC GUI → Wireless → All APs (7 joined) |
| 4 | VLAN configuration on a switch | 'show vlan brief' on SW-Core-1 |
| 5 | DHCP pool + client assignment | SRV-Core Services → DHCP AND a client's IP config |
| 6 | Wireless client pinging internal host | Laptop-IT-01 command prompt after ping |
| 7 | ACL isolating guest | 'show ip access-lists' on SW-Core-1 AND guest failing to ping employee |
| 8 | Additional features — at least two | Port security + HSRP + RADIUS + NTP (capture multiple) |

### Extra enterprise screenshots (huge boost to your Technology Descriptions section)

| Command | On device | Shows |
| --- | --- | --- |
| show standby brief | SW-Core-1 | HSRP active/standby state across all VLANs |
| show etherchannel summary | SW-Core-1 | Three EtherChannels bundled with LACP |
| show ip ospf neighbor | R-Edge | OSPF adjacencies with both cores in FULL state |
| show ip route ospf | R-Edge | All internal subnets learned via OSPF (ECMP) |
| show port-security interface fa0/1 | SW-F1 | Port security armed, sticky MAC learned |
| show spanning-tree root | SW-Core-1 | STP root bridge placement per VLAN |
| show ntp associations | Any switch | Time sync established with SRV-AAA |
| show logging | Any switch | Syslog messages timestamped and sent to SRV-AAA |

### Steps to capture good screenshots

- Use the OS native screenshot tool. Windows: Win+Shift+S. macOS: Cmd+Shift+4.

- Save each PNG with a descriptive filename: 01-topology.png, 02-ssids.png, and so on.

- Put all screenshots in a 'screenshots/' folder alongside your .pkt file.

- Zoom in if text is too small to read. Take multiple crops of the topology if it does not fit on one screen legibly.

Watch out Packet Tracer's 'Show All Labels' toggle (View menu) makes every device name visible at once. Turn it on before the topology screenshot.

# 4 · Troubleshooting

The problems below account for roughly 90% of what goes wrong in builds at this scale. Each entry has a symptom, a diagnosis, and a fix.

## 4.1 HSRP stays stuck in 'Init' or 'Listen'

Symptom: 'show standby brief' shows state Init or Listen instead of Active/Standby.

Diagnosis: HSRP hellos cannot reach the peer. Either the SVI is shut down, the inter-core trunk doesn't carry that VLAN, or the VLAN isn't defined on both cores.

Fix: Check 'show interfaces vlan X' is up/up on both cores. Check 'show interfaces trunk' includes the VLAN. Re-create the VLAN if it's missing.

## 4.2 EtherChannel doesn't bundle

Symptom: 'show etherchannel summary' shows ports as individual links, or Po state is 'SD' (suspended).

Diagnosis: Modes mismatch between the two sides, or one member link is down, or the member ports have inconsistent config (e.g., one is trunk, one is access).

Fix: Verify both ends use 'channel-group X mode active' (LACP). Verify both member ports have identical switchport config. Shut/no-shut the Po interface.

## 4.3 OSPF stuck in EXSTART

Symptom: Neighbor state shows EXSTART/EXCHANGE and never reaches FULL.

Diagnosis: MTU mismatch or router-id conflict.

Fix: Check router-ids are unique ('show ip ospf'). Set 'ip mtu 1500' on both sides if defaults differ (rare in PT but possible).

## 4.4 DHCP works on V99 but nothing else

Symptom: APs get an IP (they are on V99), but laptops on V10 never get one.

Diagnosis: Missing 'ip helper-address' on the VLAN 10/20/30 SVIs.

Fix: Add 'ip helper-address 192.168.99.40' to every user-VLAN SVI on BOTH cores.

## 4.5 802.1X association fails

Symptom: Laptop sees TechNova-Employee but cannot authenticate.

Diagnosis: Usually one of three things — wrong shared secret between WLC and RADIUS, user not defined on the AAA server, or WLC management IP not registered as a RADIUS client.

Fix: Verify all three on SRV-AAA → Services → AAA. Also confirm the WLC's Security → AAA → RADIUS entry has the correct server IP.

## 4.6 Guest SSID not broadcasting

Symptom: Phone cannot see TechNova-Guest.

Diagnosis: WLAN not enabled, or Broadcast SSID unchecked.

Fix: WLANs → edit WLAN 2 → General tab → Status: Enabled, Broadcast SSID: checked.

## 4.7 AP will not join WLC

Symptom: AP status shows 'Unassociated' indefinitely.

Diagnosis: AP cannot reach the WLC management IP on VLAN 99. Either no DHCP on V99, no L2 path from AP's switch port to WLC, or controller IP misconfigured on the AP.

Fix: Check DHCP pool has addresses available. Check AP's switchport is access V99 with PortFast. From the AP CLI, 'ping 192.168.99.10' — must succeed.

## 4.8 Config lost on reboot

Symptom: After reopening the .pkt file, devices have factory defaults.

Diagnosis: You did not 'write memory' after configuring, or you did not save the Packet Tracer file itself.

Fix: Make 'write memory' (or 'copy run start') a reflex after each phase. File → Save the .pkt file after each phase too.

# 5 · Submission checklist

Work through every line on this checklist before clicking Submit on the school portal. A failed file naming check is a 5% penalty for free.

## 5.1 File readiness

- Close and reopen your .pkt file. Verify the topology loads cleanly and nothing has reverted.

- Run the full Verification sheet one more time. All 25 tests pass.

- File name is exactly: Ssendiwala_Craig_COMP4358_FinalProject.pkt.

- PDF report exported from Word → File → Save As → PDF. Name exactly: Ssendiwala_Craig_COMP4358_FinalProject.pdf.

## 5.2 Report readiness

- Title page has name, student ID, course, professor, project title, submission date.

- Table of contents is populated (right-click → Update Field in Word).

- All 11 subsections of brief Section 4 are present.

- All 8 required screenshots included with Figure N: caption style.

- Extra enterprise screenshots included to reinforce the Technology Descriptions section.

- Bluetooth conceptual section (Section 4.7) written — at least half a page.

- References section (Section 4.11) includes at least 5 APA-style citations.

## 5.3 Submission

- Log into the NAU portal. Navigate to COMP 4358 → Assignments → Final Project.

- Upload the .pkt first, then the .pdf. Do not 'Save Draft' — click Submit.

- Portal shows a confirmation with timestamps for both files. Screenshot the confirmation page.

- Email the confirmation screenshot to yourself so it's timestamped independently. This is your proof if anything goes sideways.

Watch out If the portal only accepts one file at a time, upload the .pkt, then edit the submission to add the .pdf. Confirm both attachments show on the final submission record before walking away.

# Appendix A · Full CLI reference

A consolidated paste-ready reference. Every command here is also embedded in the phase it belongs to — the appendix is for when you want to see one device's full config in one place.

## A.1 R-Edge — full running config

hostname R-Edge

ip domain-name technova.local

username admin privilege 15 secret Techn0va!

crypto key generate rsa modulus 1024

ip ssh version 2

!

interface gi0/0

description WAN-TO-ISP

ip address dhcp

ip nat outside

no shutdown

interface gi0/1

description TO-SW-CORE-1

ip address 10.0.0.1 255.255.255.252

ip nat inside

no shutdown

interface gi0/2

description TO-SW-CORE-2

ip address 10.0.0.5 255.255.255.252

ip nat inside

no shutdown

!

router ospf 1

router-id 1.1.1.1

passive-interface default

no passive-interface gi0/1

no passive-interface gi0/2

network 10.0.0.0 0.0.0.3 area 0

network 10.0.0.4 0.0.0.3 area 0

default-information originate always

!

access-list 10 permit 192.168.0.0 0.0.255.255

ip nat inside source list 10 interface gi0/0 overload

!

ntp server 192.168.99.41

logging host 192.168.99.41

logging trap informational

service timestamps log datetime msec

service password-encryption

banner motd # Authorized access only. TechNova Inc. #

line vty 0 4

login local

transport input ssh

## A.2 SW-Core-1 — key config blocks

hostname SW-Core-1

ip routing

spanning-tree vlan 10,30 priority 4096

spanning-tree vlan 20,99 priority 8192

!

vlan 10

name Employee

vlan 20

name Guest

vlan 30

name IoT

vlan 99

name Management

!

interface gi0/1

no switchport

ip address 10.0.0.2 255.255.255.252

!

interface range fa0/23 - 24

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 1 mode active

!

interface range fa0/1 - 2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 10 mode active

!

interface range fa0/5 - 6

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 30 mode active

!

interface vlan 10

ip address 192.168.10.2 255.255.255.0

ip helper-address 192.168.99.40

standby version 2

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

interface vlan 20

ip address 192.168.20.2 255.255.255.0

ip helper-address 192.168.99.40

ip access-group GUEST-ISOLATION in

standby version 2

standby 20 ip 192.168.20.1

standby 20 priority 100

interface vlan 30

ip address 192.168.30.2 255.255.255.0

ip helper-address 192.168.99.40

standby version 2

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

interface vlan 99

ip address 192.168.99.2 255.255.255.0

standby version 2

standby 99 ip 192.168.99.1

standby 99 priority 100

!

ip access-list extended GUEST-ISOLATION

deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

deny ip 192.168.20.0 0.0.0.255 192.168.99.0 0.0.0.255

permit ip 192.168.20.0 0.0.0.255 any

!

router ospf 1

router-id 2.2.2.2

passive-interface default

no passive-interface gi0/1

network 10.0.0.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.99.0 0.0.0.255 area 0

!

ntp server 192.168.99.41

logging host 192.168.99.41

logging trap informational

service timestamps log datetime msec

SW-Core-2 is identical except: router-id 3.3.3.3, gi0/1 IP = 10.0.0.6/30, STP priorities swapped (priority 4096 for VLANs 20,99 and 8192 for 10,30), HSRP priorities swapped (110 on V20/V99, 100 on V10/V30), OSPF network 10.0.0.4 0.0.0.3, and the EtherChannel uplink to SW-F2 uses fa0/3-4 as channel-group 20.

## A.3 SW-F1 — representative floor switch

hostname SW-F1

!

vlan 10

vlan 20

vlan 30

vlan 99

!

interface range fa0/23 - 24

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

channel-group 10 mode active

interface port-channel 10

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

interface fa0/22

description BACKUP-TO-CORE-2

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99

!

interface range fa0/1 - 5

description AP-PORT

switchport mode access

switchport access vlan 99

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 2

switchport port-security violation restrict

switchport port-security mac-address sticky

!

interface vlan 99

ip address 192.168.99.4 255.255.255.0

no shutdown

ip default-gateway 192.168.99.1

!

ntp server 192.168.99.41

logging host 192.168.99.41

logging trap informational

SW-F2 and SW-F3 use the identical template. Differences per switch: hostname, the port-channel number (20 for F2 → channel-group 20, 30 for F3 → channel-group 30), the management IP (.5 for F2 and .6 for F3), and on the matching core side, the port range that joins each EtherChannel (SW-F2 → SW-Core-2 fa0/3-4; SW-F3 → SW-Core-1 fa0/5-6). On all three floor switches the uplink uses fa0/23-24 for the primary Po members and fa0/22 as the backup trunk — that part is identical.

# About this document

Generated as a companion deliverable for Craig Ssendiwala's COMP 4358 final project. All content reflects the enterprise TechNova build as designed on April 22, 2026. The scenario, VLANs, SSIDs, and security modes follow the COMP 4358 brief by Professor Azamat Zhamanov; the HA, routing, and operability features are additions explicitly permitted (WPA2-Enterprise) or listed as extra-credit in Section 3.5 of the brief (NTP, Syslog, AAA/RADIUS).

Companion files:

- TechNova_IPPlan.xlsx — reference spreadsheet with VLANs, device inventory, IP plan, port map, channel plan, and verification tests.

- technova_enterprise.html — visual mockup of the final topology.

Good luck. Build it in order. Save often.

— End of build guide —